# Tata Steel Equipment Maintenance Manual

## Engineering & Maintenance Department | 2024

---

### Section 1: Maintenance Philosophy

Tata Steel follows a Total Productive Maintenance (TPM) approach combining Preventive, Predictive, and Corrective maintenance strategies to maximize equipment availability while minimizing unplanned downtime and cost.

**Maintenance Goals:**

- Equipment availability above 95% for critical assets

- Reduce unplanned downtime by 20% year-on-year

- Mean Time Between Failures (MTBF) improvement

- Mean Time To Repair (MTTR) reduction

---

### Section 2: Maintenance Schedule Types

**Preventive Maintenance (PM):**

- Scheduled at fixed intervals based on manufacturer recommendations and operating history

- Daily, weekly, monthly, quarterly, and annual checks

- Prevents unexpected breakdowns through planned servicing

- Logged in Computerized Maintenance Management System (CMMS)

- Includes lubrication, cleaning, calibration, and component replacement on schedule

**Predictive Maintenance (PdM):**

- Based on continuous sensor data monitoring (temperature, vibration, pressure, current)

- Condition-based triggers rather than fixed schedules

- Maintenance triggered when readings cross warning thresholds

- Reduces unnecessary maintenance on healthy equipment

- Techniques used: vibration analysis, thermography, oil analysis, ultrasonic testing

**Corrective Maintenance (CM):**

- Performed after failure occurs (reactive)

- Emergency response within 2 hours for critical equipment

- Root cause analysis (RCA) mandatory for repeat failures

- Failure report filed within 24 hours on CMMS

- RCA techniques: 5-Why analysis, Fishbone diagram

**Shutdown Maintenance:**

- Planned annual/bi-annual shutdowns for major overhauls

- Requires detailed work order planning 3 months in advance

- Critical spares procurement initiated 6 months ahead

---

### Section 3: Equipment Codes & Standards

| Equipment | Code | PM Frequency | Critical Level | MTBF Target |

|---|---|---|---|---|

| Crusher Machine | EQ001 | Weekly | High | 30 days |

| Conveyor Belt | EQ002 | Daily | High | 45 days |

| Heating Regulator | EQ003 | Monthly | Medium | 60 days |

| Control Room Systems | EQ004 | Monthly | Critical | 90 days |

| Locomotive | EQ005 | Weekly | High | 30 days |

| Blast Furnace | EQ006 | Daily | Critical | 365 days |

| Battery (Coke Oven) | EQ007 | Daily | Critical | 180 days |

| Stacker/Reclaimer | EQ008 | Weekly | Medium | 45 days |

---

### Section 4: Troubleshooting Guide

**Crusher Machine (EQ001):**

Problem: Abnormal vibration

Possible causes:

- Worn out bearings

- Loose bolts/foundation

- Material blockage in crushing chamber

- Belt or coupling misalignment

- Rotor imbalance

Immediate steps:

- Stop machine immediately

- Lock out power supply (LOTO procedure)

- Check for visible blockage and clear if safe

- Inspect bearing condition (heat, noise, play)

- Check foundation bolts for looseness

- Call maintenance team if unresolved within 30 minutes

Problem: Overheating

Possible causes:

- Insufficient lubrication

- Overloading beyond rated capacity

- Cooling system failure

- Bearing degradation

Steps:

- Reduce load immediately

- Check lubrication levels and quality

- Inspect cooling system (fans, water circulation)

- Allow to cool 30 minutes before restart

- If recurring, schedule bearing inspection

Problem: Reduced output / capacity drop

Possible causes:

- Worn crushing plates/jaws

- Incorrect feed size

- Belt slippage on drive

Steps:

- Check feed material size against spec

- Inspect crushing plate wear (measure against tolerance)

- Check drive belt tension

**Conveyor Belt (EQ002):**

Problem: Belt slipping

Steps:

- Check belt tension against specification

- Inspect drive pulley for wear/lagging damage

- Check for material spillage causing buildup

- Adjust tension if needed using takeup mechanism

- Check for moisture/contamination on pulley surface

Problem: Unusual noise

Steps:

- Identify noise source (idler, pulley, or belt)

- Check all rollers/idlers for bearing wear

- Inspect belt for damage (tears, splice failure)

- Check lubrication of bearings

- Verify belt tracking/alignment

Problem: Belt tracking off-center

Steps:

- Check idler alignment along entire conveyor length

- Inspect for uneven loading causing belt drift

- Adjust training idlers

- Check structure for tilting/settling

**Heating Regulator (EQ003):**

Problem: Temperature fluctuation beyond setpoint

Steps:

- Check fuel supply pressure and flow

- Inspect thermocouple/sensor calibration

- Verify control valve operation

- Check for air leaks in combustion chamber

- Review control logic settings in SCADA

Problem: Error code displayed

Steps:

- Note exact error code and timestamp

- Refer to equipment-specific error code manual

- Check sensor wiring connections

- Restart controller if safe (follow restart procedure)

- Escalate to control system team if error persists

**Locomotive (EQ005):**

Problem: Brake system warning

Steps:

- Stop locomotive in safe location

- Do not proceed until brake system inspected

- Check air pressure gauges

- Inspect brake pads/shoes for wear

- Contact rolling stock maintenance team

---

### Section 5: Sensor Thresholds

| Sensor | Normal Range | Warning | Critical |

|---|---|---|---|

| Temperature | 20-80°C | 81-95°C | Above 95°C |

| Vibration | 0-0.5 mm/s | 0.5-1.0 mm/s | Above 1.0 mm/s |

| Pressure | 100-130 PSI | 131-150 PSI | Above 150 PSI |

| RPM | 800-1200 | 1201-1400 | Above 1400 |

| Oil Temperature | 40-70°C | 71-85°C | Above 85°C |

| Current Draw | Rated ±10% | Rated ±20% | Rated ±30% |

**Action Matrix:**

| Status | Action Required | Timeline |

|---|---|---|

| Normal | Continue routine monitoring | - |

| Warning | Schedule inspection, increase monitoring frequency | Within 48 hours |

| Critical | Stop equipment, immediate maintenance intervention | Immediate |

---

### Section 6: Spare Parts Management

- Critical spares maintained at minimum stock levels defined per equipment

- Reorder triggered automatically when stock falls below reorder point

- Long lead-time items (bearings, motors, control cards) reviewed quarterly

- Obsolete equipment spares cross-referenced for alternate suppliers

| Spare Category | Examples | Min Stock |

|---|---|---|

| Bearings | Crusher, conveyor bearings | 4 units |

| Belts | Conveyor belts, V-belts | 2 units |

| Sensors | Temperature, vibration probes | 6 units |

| Electrical | Contactors, relays, fuses | 10 units |

---

### Section 7: Permit to Work System

All maintenance work requires appropriate permit:

| Permit Type | Required For |

|---|---|

| Hot Work Permit | Welding, cutting, grinding |

| Confined Space Permit | Tank, vessel, pit entry |

| Electrical Work Permit | Any electrical isolation/work |

| Excavation Permit | Digging near underground services |

| Working at Height Permit | Work above 1.8m |

---

### Section 8: Emergency Maintenance Contacts

| Role | Contact |

|---|---|

| Maintenance Supervisor | Extension 301 |

| Electrical Team | Extension 302 |

| Mechanical Team | Extension 303 |

| Control Room | Extension 304 |

| Instrumentation Team | Extension 305 |

| Emergency Response | Extension 999 |